# Instructions to run the DQN Accelerator on the ZCU104 Board

This guide explains how to prepare the ZCU104 board, transfer the project, load the FPGA accelerator, and run the Python application

Requirements:

- Xilinx ZCU104 board with microSD Card

- Computer connected to the same network as the board

- A PYNQ image compatible with the ZCU104 board

- Project files including the FPGA accelerator (dqn_eva.bit and dqn_eva.hwh files) and Python Environment Script (processor.py)

1.  Flash the PYNQ Image to the microSD Card

- Download the PYNQ image intended for the ZCU104

- Insert the microSD card into the host computer.

- Use a flashing utility balenaEtcher to write the image file to the card.

- Safely eject the card after flashing is complete.

- Insert the microSD card into the ZCU104 SD-card slot.

2. Connection to the Board

- Set the ZCU104 boot-mode switches for SD-card boot according to the board documentation.

- Connect the power supply and turn on the board.

- Connect Ethernet between the board and the local network.

- Determine the board IP address from the router, serial console, or network scan(Advanced IP Scanner).

- Use MobaXterm application to connect to SSH.

3. Copy the necessary files to the board.

- Copy the FPGA bitstream (dqn_eva.bit) and matching hardware description (dqn_eva.hwh) in the overlays folder (/home/xilinx/pynq/overlays/dqn_eva/).

- Copy the python script (processor.py) in the main folder (/home/xilinx/).

4. Python file run menu

- After copying the files to the board, run the following command in MobaXterm

cd /home/xilinx

sudo python3 processor.py

- After the overlay is loaded, the program displays two options:

1 = Train DQN and save weights

2 = Load saved weights and run inference/evaluation.

5. Training mode

- Enter 1 to run the training mode.

- Training runs using the complete episode configuration defined in processor.py.

- After training, the program saves CSV result files in: /home/xilinx/dqn_agent_results

- And the trained network weights are stored in: /home/xilinx/trained_dqn_weights.npz

6. Inference mode

- Enter 2 to load the saved weight file and run inference.

- If no valid saved-weight file is available, the program automatically switches to training mode. If a valid recovery or backup weight file is available, it may use that file instead and runs inference